Date____1/15/16________

# of test cases created:

0

# of test cases performed:

0

What I learned:

Other :

Continued work on the transcend working on preset settings and other settings using the kent build for aa. Updated Kevin on the bug I reported concerning the parameter preset group third party device assignment. After further testing he found additional problems with parameter groups names and other panel devices missing that should be listed in the venue explorer and should match each panel name on aa so he changed that particular bug status to blocker .

Had a mtg with Jim discussing the current bug list trying to find out which bugs are the most critical at this point. He wanted us to install the lex dev version of aa and to check the preset changes while going off line and then back online checking for a possible ghost recall or store happening which is the first bug on his list. Changed the aa software to lex dev v 81 and began testing presets while going offline and back online. I determined that aa seems to connect to the transcend box when going online but does not collect its info until a recall has been performed also, if a change is made to a preset but not saved and then going offline and back online causes the save function to fail until another preset is recalled. This does appear like it could be a ghost store but it is my opinion that it just looks like that but in reality it does not restore or recall at all but instead copies the last panel change and retains it when changed from offline to online status. Due to all of this shifting around I created an outline detailing what I know about the box currently including the panel and device presets and their functionality just to keep things as straight as possible.

Received more wall controllers. Installed them and then connected them to aa. Had to change their auto ip to a static ip address and then aa found them. Created a parameter preset group and assigned presets to at least one controller.

Received a venu 360 dante box which I connected and began checking its functions had to create an ip address and enter it manually, after talking with jeff this is how the dante box works. Opened the venu app and connected the box then connected dante controller. Created an audio path making sure dante controller was allowing the boxes to route signals.

Upgraded the transcend firmware to v 23.3 and am now testing the zone presets recently added using the web interface only.

Attended an engineers mtg and a dv mtg as well.