Date____3/4/16________

# of test cases created:

# of test cases performed:

Other :

Spent time performing regression tests on the transcend box using audio architect v 96 testing both the zone and device presets with various configurations while saving and storing changes. Created eq patterns on each panel reflecting different parameters while using svs. Still wondering why I am asked to save a change before changing a preset when no change was made, this seems to be confusing. Added and changed values for each panels relating to each preset so I would recognize an inconsistency once a preset was recalled.

Attended a transcend mtg where the issue with the box failing to be discovered is being raised up again. Mike should be releasing a new version of firmware soon which needs to be tested for this issue. Received the firmware for the transcend box v 1.02 and ran a test overnight. The test started at approx 5:35 and failed at 6:40. Reran the test the next day to be sure and it failed again. Sent a copy of putty to Mike and he thinks that aa is causing this. Talked with Kevin and he decided to remove his box from the network and after slowing eggplant functionality the test results were improving.  Decided to lengthen the test a little bit more by adding delay times to the script and the test improved and actually completed all cycles. Attended another mtg and gave the results. Jim now wants me to remove the delay times slowly speeding up the tests to see if it fails again like it failed before.

Some new issues appear to be happening with panel preset recall and store setting which needs to be tested. Verified Ron’s complaint that recalled eq patterns disappear after switching from auto eq to manual and then flat back to manual soon the signal is gone.

Informed Jim that I performed regression testing using aa v 96 and firmware v 1.01 on Monday.

Upgraded aa to v1.90.0.97.

Received instructions to begin work on a bug for dbx venu360 for noise occurring on outputs 2, 3and 4 when using a 96k sample rate. Talked with John to get more detail and then using the venu360 desktop app, updated one box to firmware to v 2.05 while having the another venu box set with firmware version 2.04. Then listened to each output of each box one at a time comparing differences between each output used in 20 presets. Narrowed down to the presets having the noise which are 2, 3, 7, 8, 9, 10, 11 and 13. Gave John a report on those results. Continued testing this box but using audio input routing and checking for the results using an output cable using a/b testing on both boxes with different versions of firmware.

Attended a design validation mtg.

Del sent me an e-mail requesting the license key for the eggplant software. After he received it we talked about how things should be arranged to get the best use out of it. Tony also requested a license key.

Contacted DJ to schedule another mtg for eggplant software which is Monday March 7 11:30 at the red rocks conference room.